Name Tag - Beginner Exercise Steps

- Start a NEW file using the ANSI Metric Part.par template.

- Show the Base Reference Planes

- Create a Rectangle By Center on the Top Plane > 100 mm wide x 50 mm high

- Hide the Reference Planes

- Extrude the region 10 mm into the screen (-Z direction)

- Add 10 mm rounds to the four corners of the block

- Create a Thin Wall that is 4 mm thick and open the top face

- Add 1.5 mm rounds to the outside and inside edges of the block

- Sketch a 5 mm diameter circle centered vertically on the inside face and add a Smart dimension to locate it 14 mm from the outside edge.

- Drag the region through the part to create a hole

- Add a Text profile sketch on the inside face of the tag

  - Set to a bold Font (Barlow Condensed ExtraBold)

  - Height of 12 mm

  - Change the anchor point to center

  - Place near center of the inside face

  - Use Horizontal/vertical relationship to center the text vertically with the hole

- Use the Extrude command with single or chain option to select the text profile and extrude out 2 mm.

- Use Part Painter to change colors:

  - Body option for the first color

  - By Feature for the second color to paint the text something different

- Use synchronous technology to:

  - change the width of the tag.

  - change the angle of the short side

  - move the hole to a corner radius center point

  - change the angle of the other short side

- Save the file

- Use the 3D Print option under the Application Button to generate an STL file for printing.